MRI Slice Reconstruction — Next Steps

Research Discussion Document

Prepared for group discussion

# 1. Clinical Viability & Real-World Relevance

The core idea of acquiring fewer MRI slices and reconstructing the missing ones with AI has genuine clinical value. The research demonstrated that a U-Net model trained on ×2 acceleration (1 missing slice per gap) can reconstruct missing slices with acceptable quality (PSNR ~31 dB, SSIM ~0.90), which is competitive with classical baselines.

## 1.1 Why This Matters Clinically

- MRI scans typically take 20–45 minutes — AI-assisted acceleration could halve this time or use saved time to acquire additional contrast sequences (T1, T2, FLAIR).

- Particularly impactful for claustrophobic patients, children, and patients who struggle to remain still.

- Industry is already moving this direction: Siemens (Deep Resolve), GE (AIR Recon DL), and Philips (SmartSpeed) all have FDA-cleared AI reconstruction products in clinical use.

- The explainability component adds a key differentiator — radiologists are skeptical of AI black boxes, and interpretable outputs build trust.

## 1.2 Key Limitation: Lesion Risk

The most serious clinical concern identified is the risk of missing pathology in reconstructed slices:

A small lesion (microbleed, MS plaque, early metastasis) existing entirely within one missing slice could be averaged away by the reconstruction model. The model has no way to detect something abnormal should be present because it was never trained on pathological cases. The smaller the lesion relative to slice thickness, the higher the risk.

This is not a blocker but a constraint: reconstructed slices should not be used as the sole basis for diagnosis without additional safeguards (see Section 2 — Adaptive Acquisition).

## 1.3 Current Dataset Limitation

The IXI dataset consists entirely of healthy volunteers. This means:

- The reconstruction model has only seen healthy anatomy and will confidently produce normal-looking tissue even when pathology is present.

- No pathological examples exist to train an anomaly detector.

Datasets needed to extend this work:

| Dataset | Pathology | Use Case |
| --- | --- | --- |
| BraTS | Brain tumours | Reconstruction + anomaly detection training |
| MSLUB / MSLesSeg | MS lesions | Small lesion detection validation |
| ADNI | Alzheimer's (subtle atrophy) | Subtle pathology reconstruction |

# 2. Adaptive Acquisition — The Key Next Step

The most clinically valuable extension identified is an adaptive scanning pipeline: acquire half the slices first, run anomaly detection, and only acquire the remaining slices if something suspicious is found.

## 2.1 Proposed Pipeline

| Phase | Action | Outcome |
| --- | --- | --- |
| Phase 1 — Fast Scout | Acquire every 2nd slice (50% of data) | Scan time halved |
| Phase 2 — Detection | Run anomaly detection on acquired slices | Normal vs suspicious classification |
| Phase 3a — Normal | Reconstruct missing slices with U-Net | Full volume from AI, fast scan complete |
| Phase 3b — Suspicious | Acquire all remaining slices normally | Full real scan, no information lost |

## 2.2 Why This Is Viable

- The edges and background of a brain MRI are anatomically simpler than the center — AI reconstruction is easier where the scanner is spending least effort.

- The model only needs to be correct in the common case (healthy/normal anatomy) — safety is guaranteed by fallback to full acquisition.

- Uncertainty quantification (see Section 3) provides a natural gate: high model uncertainty triggers full acquisition automatically.

## 2.3 Components Required

- Anomaly detector: trained on paired healthy/pathological scans with lesion masks.

- Uncertainty estimator: Monte Carlo Dropout (already implementable in the current U-Net).

- Acquisition controller: hardware-level integration with scanner software — the most challenging component for a research group.

This idea is novel enough to be a distinct research contribution. The framing of 'conditional full acquisition triggered by AI uncertainty' does not appear to be widely implemented in the published literature and could form the basis of a follow-up paper.

# 3. Explainable AI — Extended Methods

The current implementation uses Grad-CAM and Integrated Gradients. The following additional XAI methods were identified as high-value extensions, ranked by clinical relevance and implementation effort.

## 3.1 Priority Recommendations

| Method | Clinical Value | Effort | Novelty |
| --- | --- | --- | --- |
| MC Dropout Uncertainty | Very high — directly supports adaptive acquisition | Low | Medium |
| Slice Contribution Analysis | Very high — unique to interpolation task | Very low | High |
| Attention Maps (Transformer) | High — intrinsic, not post-hoc | Medium (needs retraining) | Medium |
| SHAP | High — mathematically guaranteed attribution | Low | Low |
| Occlusion Sensitivity | High — most intuitive for clinicians | Low | Low |

## 3.2 Monte Carlo Dropout Uncertainty

Add dropout to the U-Net bottleneck and run 30 forward passes at inference time. The standard deviation across passes produces an uncertainty map — regions of high variance indicate where the model is least confident.

The critical clinical question this answers: does high uncertainty correlate with high reconstruction error? If yes, the model knows when it does not know — the foundation of a safe adaptive scanning system.

This is the single highest-priority addition to the explainability pipeline because it directly connects to the adaptive acquisition idea in Section 2.

## 3.3 Slice Contribution Analysis

Blank out each input slice in turn and measure how much the prediction changes at each spatial region. This produces a left-contribution map and a right-contribution map showing which parts of the reconstruction are driven by which neighbor.

This method is unique to the interpolation task and cannot be found in general XAI toolkits — it is a genuine novel contribution. The dominance map (left minus right) reveals asymmetries in how the model uses its two inputs, which has direct implications for acquisition order.

## 3.4 LLM-Powered Medical Interpretation

The current implementation already supports automated medical interpretation of explainability outputs using the Anthropic Claude API. Each sample's Grad-CAM and IG maps are sent alongside quantitative metrics and the model returns a structured clinical assessment covering:

- Reconstruction quality assessment and clinical acceptability.

- Anatomical structure identification from Grad-CAM.

- Input reliance analysis (left vs right slice symmetry).

- Error localisation and clinical significance.

- Clinical concern level rating (LOW / MEDIUM / HIGH).

This component transforms the system from a technical tool into something a radiologist can interact with and interrogate — a significant step toward clinical deployment.

# 4. Reduced Field-of-View Acquisition

A second novel idea was proposed: the scanner acquires only the central region of interest at full resolution, and the AI reconstructs the peripheral edges. This is a spatial variant of the temporal interpolation already implemented.

## 4.1 Rationale

- The edges of a brain MRI are mostly skull, scalp, and background — anatomically simpler than the center where the cortex, ventricles, and deep structures live.

- This means the reconstruction task is easier at the edges than in the center — the opposite of where the scanner spends its time.

- A sensible division of labour: the scanner does the hard part (center), AI does the easy part (edges).

## 4.2 Relationship to Existing Techniques

Reduced Field-of-View (rFOV) acquisition already exists in clinical MRI — scanners focus excitation pulses on specific regions. The novel contribution here is using AI to fill in the unacquired periphery rather than simply discarding it.

## 4.3 Key Technical Challenge

The boundary between the real scan region and the reconstructed region must be handled carefully — any visible seam would be clinically unacceptable. The model would need to be trained specifically on this task (partial FOV input, full FOV target) rather than adapted from the slice interpolation model.

# 5. Summary — Prioritised Next Steps

| Priority | Action | Dependency | Impact |
| --- | --- | --- | --- |
| 1 — Immediate | Add MC Dropout uncertainty to current model | None — current model | Enables adaptive acquisition gate |
| 2 — Immediate | Implement slice contribution analysis | None — current model | Novel XAI contribution |
| 3 — Short term | Acquire pathological dataset (BraTS/MSLUB) | Data access | Enables anomaly detector training |
| 4 — Short term | Train anomaly detector on pathological data | Priority 3 | Core of adaptive pipeline |
| 5 — Medium term | Implement full adaptive acquisition pipeline | Priorities 3 + 4 | Main clinical contribution |
| 6 — Medium term | Add attention layer to U-Net bottleneck | Retraining required | Intrinsic XAI — stronger claims |
| 7 — Long term | Reduced FOV spatial reconstruction | New training data | Novel acquisition paradigm |

Note: All results reported in this document are based on the IXI dataset (healthy volunteers only). Clinical validation on pathological cases is required before any of these methods could be considered for deployment. Regulatory approval (FDA, CE) requires extensive prospective clinical trial evidence.